Fecal Glucocorticoid Metabolite (FGM) Analyses Protocol

Materials

- ImmunoChem Double Antibody Radioimmunoassay Kit (MP Biomedicals, Orangeburg, NY, CAT # 07120103)

- Lyophilizer

- -80 °C freezer

- -20 °C freezer

- Pipets

- Gamma counter

- Vortex mixer

- Test tube rack

- Centrifuge

- 10 x 75 tubes for RIA

- Absorbent paper for blotting

- Mortar and pestle

- 90% methanol

- Ethanol to clean mortar and pestle

- Volunteers to assist with grinding samples

- Test tubes for storage of ground and extracted fecal samples

Methods

Extraction

- All samples should be prepared and extracted prior to moving on to the assay portion of the protocol. The number of samples prepared at a time will be restricted to the number of samples the vortex or centrifuge may hold.

- Remove fecal samples for analyses from -80 °C freezer. Samples may be refrozen in a     -20°C non-defrosting freezer after stopping points and for long-term storage.

- Samples must be freeze-dried using a lyophilizer for 24 hours to control fiber and water content.

- Dried samples must then be homogenized through grinding the samples together with a mortar and pestle. Samples must be finely ground and should be ground for roughly 4 minutes. Mortar and pestle will be cleaned with ethanol between each sample to decrease cross-contamination. Procedure can come to a full stop after this step if needed.

- After the samples have been ground, weigh out 50mg of sample for extraction. Remainder of ground samples left can be frozen for long-term storage.

- Mix the extracted feces with 1ml of 90% methanol for 30 minutes on a shaker at 1,800rpm.

  - Place saran wrap over tops of tubs to ensure no methanol is spilled during vortex.

  - Vortex speed should be as fast as it can go without “spilling” out the methanol.

- Centrifuge samples for 20 minutes at 14,000rpm.

  - Pipette out supernatant into new labeled tubes for storage.

    - This supernatant solution is what the assay will be run on.

  - Procedure can come to a full stop and samples frozen at -20°C after this step if needed.

Assay

- Quantify corticosterone using the ImmunoChem Double Antibody Radioimmunoassay (RIA) kit (MP Biomedicals, Orangeburg, NY, CAT # 07120103)

- Prior to running official samples, five samples (from any individual from the control period of the validation study or Di-Lane samples) should be selected, pooled, and serially diluted to run a series of assay dilutions on to determine the proper assay dilution rate for the study system.

- Remainder of assay will follow manufacturer kit instructions.