DNA Salt Extraction Protocol

Materials Needed:

TKM Buffer (see recipe)

Proteinase K solution (Qiagen Cat.# 19131)

6M NaCl

20% Sodium Dodecyl Sulfate (SDS) – VWR Catalog # - EM-7990

100% ETOH

70% ETOH

TE Buffer – Invitrogen Catalog # 12090-015 or can make a standard TE buffer

Recipe for TKM Buffer (to make 1 Liter)

10mM Tris, pH 7.6 (FW = 121.1)	=	1.21g Tris

10mM KCL (FW = 74.56)		=	0.75g KCL

2mM EDTA (FW = 372.2)		=	0.74g EDTA

4mM MgCl2 (FW = 95.22)		=	0.38g

dH20					=	1 liter

Recipe for Saturated 6M NaCL (FW = 58.44)

100ml water + 35.0g NaCL

*If salt crystallizes out overnight, remake.  This doesn’t work as well.

- If you are using tissue, weigh out about 50mg of tissue and homogenize with a glass rod in 200ul TKM buffer until the solution looks milky. Liver or spleen works best. If you are doing germinal discs, GDs should have been stored in 70% ETOH. Before extraction, centrifuge and resuspend the pellet in 200ul TKM buffer.

- Add 20ul proteinase K and 37.5ul SDS and incubate tube for 15 minutes at 56 degrees.

- Add 200ul 6M NaCl let sit for a minute and then add 300ul TKM buffer.

- Centrifuge at max speed (14,000rpm) for 10 minutes.

- Transfer 500ul of supernatant to new 1.5ml tube containing 900ul 100% ETOH. There will be some yolk floating around in there. Just do your best to avoid it.

- Centrifuge at max speed (14,000rpm) for 10 minutes.

- Pour off supernatant and add 500ul 70% ETOH.

- Centrifuge at max speed (14,000rpm) for 10 minutes.

- Aspirate off supernatant and dry pellet for an hour or two.

- Add 10-30ul TE buffer and incubate tube in heat block at 56 degrees for 10 minutes.

- Vortex really well and then quick spin to get liquid to bottom of tube before using nanodrop to quantify DNA quantities.